Step 1: Gather materials.

- Lid at least 50mm diameter

- Microphone/headphones with built-in microphone

- Tape (electrical is preferred)

- Knife or Scissors

- Permanent Marker

- Plastic bag

Step 2: Cut hole into lid.

- Ensure hole is cut large enough to fit the microphone through it.

Step 3: Thread microphone through lid and secure.

- Tape down microphone to the lid but do not cover the microphone port.

- Cover the hole completely so that once the membrane is attached, it is airtight.

Step 4: Trace a circle and cut out.

Step 5: Tape the plastic circle to the front of the lid and secure.

- Ensure the membrane is fully attached and airtight.

Step 6: Clean up and record locally or on heart.ai!